SDWRGMPD 1.10

TABLE OF CONTENTS

WHAT IS SDWRGMPD? . . . . . . . . . . . . . . . . . . . . . . . . .    2

HOW TO USE SDWRGMPD . . . . . . . . . . . . . . . . . . . . . . . .    4

OPTIONS . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    5

Help  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    6

Disable . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    6

Enable  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    7

Uninstall . . . . . . . . . . . . . . . . . . . . . . . . . . . .    7

StatusTable . . . . . . . . . . . . . . . . . . . . . . . . . . .    8

Center  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .    9

Button Mapping  . . . . . . . . . . . . . . . . . . . . . . . . .   12

OptionsFile . . . . . . . . . . . . . . . . . . . . . . . . . . .   18

THE SDWRGMPD ENVIRONMENT VARIABLE . . . . . . . . . . . . . . . . .   20

ERRORLEVELS . . . . . . . . . . . . . . . . . . . . . . . . . . . .   22

SDWRGMPD & WINDOWS  . . . . . . . . . . . . . . . . . . . . . . . .   23

OTHER DIGITAL JOYSTICKS . . . . . . . . . . . . . . . . . . . . . .   25

TROUBLESHOOTING . . . . . . . . . . . . . . . . . . . . . . . . . .   27

A WORD FROM THE SPONSOR . . . . . . . . . . . . . . . . . . . . . .   29

OTHER PROGRAMS  . . . . . . . . . . . . . . . . . . . . . . . . . .   31

WARRANTY  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   32

REVISION HISTORY  . . . . . . . . . . . . . . . . . . . . . . . . .   33

SDWRGMPD 1.10            Page 1 of 33

WHAT IS SDWRGMPD?

�����������������

SDWRGMPD is a DOS driver for a Microsoft SideWinder Digital GamePad.

When Microsoft created their line of SideWinder digital input devices,

they only wrote software drivers for Windows 95+.  If all you ever use

are Win95-native games, you don't need SDWRGMPD (the Microsoft Win95

drivers will do what you need).  Of course, there are in reality very

few Win95-native games.  Almost all games are written for DOS, not

Windows, because Windows game support, quite frankly, sucks.

You also may not need SDWRGMPD even if you use DOS programs, as long as

they are all capable of running in a DOS box underneath Win95.  The

Win95 software that comes with the GamePad tries to emulate a real

(analog) joystick in a DOS box, so the DOS program will think you have a

real joystick.  Unfortunately, the software emulation that the Windows

software provides is not very good (in fact, it's downright terrible).

Some programs will be fooled by the Windows software emulation and some

won't.

You will need SDWRGMPD if you have a SideWinder Digital GamePad, and

want to use it with a particular program, and at least one the following

are also true:

1. You don't use Win95 at all, or at least rarely.  You use DOS, or

you use an older version of Windows.

2. You use Win95, but have at least one DOS program that doesn't work

in a Win95 DOS box.  You have to boot to DOS, or have Win95

"Restart in MS-DOS mode", for the program to work at all.

3. You have a program that works in a Win95 DOS box, but the program

doesn't believe the Win95 software emulation is a real joystick.

4. You have a program that works in a Win95 DOS box, but you don't

like the way the Win95 software emulation maps the digital buttons

to the analog joystick values.  Microsoft's default mapping, in my

opinion, is not well thought out, and Microsoft does not provide a

way for you to change it.

SDWRGMPD 1.10            Page 2 of 33

There is a BIOS function built into nearly all computers that a program

can use to tell it what the coordinates of a real (analog) joystick are.

Most programs that use a joystick call this BIOS function when they want

to know which way you have the joystick handle pushed and which joystick

buttons you have pressed.  Of course, the standard BIOS function only

knows how to handle a real (analog) joystick, and does not work with any

of the new digital joysticks (like the Microsoft SideWinder products).

SDWRGMPD replaces the joystick BIOS function with one of its own.

SDWRGMPD "maps" the digital buttons from a Microsoft SideWinder digital

GamePad so that they look like analog values coming from a real

joystick.  If a program asks the BIOS about the joystick, SDWRGMPD will

make the program think you have an analog joystick.

While most programs use this BIOS function to learn about the joystick,

there are also some that don't.  Instead, they look at the joystick

themselves directly.  The way a digital joystick "talks" to the

computer, though, is very different than the way an analog joystick

does.  A program that looks directly at a digital joystick will think

the joystick is broken, unless it is specifically designed to know about

digital joysticks.  SDWRGMPD will not work with a program that looks at

the joystick directly - the program must use the BIOS to look at the

joystick.

SDWRGMPD sort of does the same thing that the Win95 software does in a

DOS box, but is much smaller, much faster, much more reliable, and

doesn't require Windows.  If you are trying to use SDWRGMPD in a DOS box

underneath Win95, there are some special considerations you need to take

care of.  This is discussed in the "SDWRGMPD & WINDOWS" section below

(page 23).

SDWRGMPD 1.10            Page 3 of 33

HOW TO USE SDWRGMPD

�������������������

The best way to use SDWRGMPD is to install it in your AUTOEXEC.BAT file.

It uses only a small amount of memory (about 1100 bytes), so you

shouldn't even notice it.  You can also load it into High Memory with

the DOS LH or LOADHI commands, which can save you even more memory.  It

can be DISABLEd (page 6) and ENABLEd (page 7) any time you want, if for

some reason you want to temporarily use an real analog joystick instead

of the GamePad.  You can also UNINSTALL it from memory completely (see

page 7).

You install SDWRGMPD into memory by simply typing SDWRGMPD {Options} at

the DOS command line.  If SDWRGMPD is already in memory, typing

SDWRGMPD {Options} will update the SDWRGMPD that is already in memory.

This allows you to change the settings for SDWRGMPD any time you want.

The default settings for SDWRGMPD should work with most programs, but

you can still change them.

You are allowed to change which digital buttons on the GamePad are

mapped to which analog values in the joystick BIOS function (see page

12).  You are also allowed to change the analog values that are returned

by the BIOS when a program asks the BIOS about the joystick coordinates

(see page 9).

If a particular program needs to have special settings for SDWRGMPD

(something other than the defaults), the best thing to do is to write a

batch file to change things for you automatically.  If you don't know

how to write Batch files, I strongly encourage you to learn - the

ability to write and use Batch files will save you literally hours of

work and frustration in years to come.  With a Batch file, you can have

SDWRGMPD automatically change the settings for a particular program, run

the program, and then change the settings back to the defaults again so

the next program that uses the GamePad will work properly.  For example,

you could write a batch file to run "Program" like this:

ECHO OFF

C:

CD \Program

SDWRGMPD /Reset {other options}

REM Reset SDWRGMPD and change things so they're not at defaults

IF ERRORLEVEL 1 GOTO ERROR

Program

SDWRGMPD /Reset

REM Reset SDWRGMPD back to defaults

GOTO END

:ERROR

ECHO Error occurred while installing/updating SDWRGMPD.

:END

CD\

SDWRGMPD 1.10            Page 4 of 33

OPTIONS

�������

SDWRGMPD is fairly liberal in what it will accept as Options.  You can

tell SDWRGMPD which Options you want it to use in three different ways:

from the DOS command line, with an Environment variable called SDWRGMPD

(see "THE SDWRGMPD ENVIRONMENT VARIABLE" below, page 20), or with a

Configuration file (see the OPTIONSFILE Option below, page 18).  The

format of three different methods of giving SDWRGMPD the Options are

very similar, but there are some differences.

All Options must be entered separately.  On the DOS command line, or

with the SDWRGMPD Environment variable, each Option must start with a

slash ("/") or a dash ("-").  In an OPTIONSFILE, each Option is put on a

separate line in the file (and they don't start with a "/" or a "-").

Options that require a number or character to be entered as part of the

Option (indicated by a ":" separator in the Option details below) can

either use a ":" or "=" as the separator.  They can be upper- or lower-

case, do not need to be separated by spaces, and can be entered in any

order.  If the same Option is entered twice with two different settings,

or conflicting Options are entered, the last one entered takes

precedence.

Note also that there are usually several different ways to enter the

same Option.  For example, to DISABLE SDWRGMPD (turn it off), you could

type any of the following from the command line:

SDWRGMPD /D

SDWRGMPD /Disable

SDWRGMPD /Off

Most of the Options have a "shorthand" code ("/D" in this case) and one

or more "longhand" codes ("/Disable" and "/Off" in this example).  The

shorthand codes take up less room and are faster and easier to type, but

the longhand codes are easier to remember and to read and to understand.

You can use whichever makes the most sense to you.

For instance, the following are equivalent:

SDWRGMPD /Enable /ALeft=ButtonB

SDWRGMPD -e      -aL   : B

Again, the format when giving SDWRGMPD Options from an OPTIONSFILE is

slightly different than when giving SDWRGMPD Options from the command

line (or from an Environment variable).  As you are reading the details

about the Options, you will usually see the command line form of the

Option (which starts with a "/"), rather than the OPTIONSFILE form of

the Option (which doesn't start with a "/").  This hopefully will not

confuse you too much.  Read the details in the OPTIONSFILE Option below

(page 18) if you want to know more.

SDWRGMPD 1.10            Page 5 of 33

* /H

* /Help /Hlp /? ?

This pulls up a HELP screen which gives a quick summary of the

possible SDWRGMPD Options.

* /D

* /Disable /Off

This DISABLEs SDWRGMPD, so that it doesn't do anything useful any

more.  There are only two times I can imagine needing to DISABLE

SDWRGMPD.  The first is if you actually have two devices, a

SideWinder GamePad and a real (analog) joystick.  When you are

using the SideWinder, you should leave SDWRGMPD ENABLEd (see page 7

below).  You can DISABLE it when you are using the real joystick

(either by unplugging the SideWinder and plugging in the real

joystick, or by pressing the {Mode} button on the SideWinder if you

have the SideWinder and the real joystick cascaded together).  You

can then re-ENABLE SDWRGMPD (page 7 below) when you want to use the

SideWinder GamePad again.

The other time you might want to DISABLE SDWRGMPD is if you are in

a DOS box underneath Win95, and want to use the software emulation

of the Microsoft software instead of using SDWRGMPD.  SDWRGMPD is

actually much faster and much more reliable than the Microsoft

emulation, but you still may want to use the Microsoft software

anyway.  This is discussed further in the "SDWRGMPD & WINDOWS"

section below (page 23).

SDWRGMPD uses such a small amount of memory, and doesn't do

anything useful until a program asks about the joystick anyway,

that leaving it ENABLEd all the time won't hurt anything or waste

any time.  If you do DISABLE SDWRGMPD, it can be reENABLEd with the

ENABLE Option (page 7 below).  SDWRGMPD can be UNINSTALLed from

memory completely with the UNINSTALL Option (see page 7 below).

SDWRGMPD 1.10            Page 6 of 33

* /E

* /Enable /Reset /On

This ENABLEs SDWRGMPD so that it is turned on (if it was DISABLEd

with the DISABLE Option, page 6 above).  It will also Reset all

changeable values in SDWRGMPD so that they are restored back to

their defaults.

There are not a lot of changeable values in SDWRGMPD.  The things

you are allowed to change are which digital buttons are mapped to

which analog values (see page 12 below), and the coordinate values

that are returned by the BIOS when a joystick handle is moved (see

page 9 below).

When SDWRGMPD is initially installed in memory, the default is for

it to be ENABLEd.

If you want to change one of more of the values in SDWRGMPD, you

should nearly always make the ENABLE Option the first Option you

enter.  This is because later Options always takes precedence over

previous Options.  Let's compare the following two lines:

SDWRGMPD /Enable        /ALeft=ButtonM

SDWRGMPD /ALeft=ButtonM /Enable

The first line will ENABLE and Reset everything back to default

values, and THEN change the mapping so that the {M} button on the

SideWinder is mapped to the Left movement of analog joystick A.

The second line maps the button, and then ENABLEs (Resets)

everything back to the default values.  By default, the {M} button

on the SideWinder is not normally mapped to the Left movement of

analog joystick A (ALeft).  The end result of the second line will

simply be to ENABLE and Reset SDWRGMPD, just as if the

"/ALeft=ButtonM" had never been entered.

* /U

* /Uninstall /Remove /Out

This will permanently UNINSTALL (or at least try to UNINSTALL)

SDWRGMPD from memory.  Effectively, the same thing can be

accomplished with the DISABLE Option above (page 6), but this

UNINSTALL option will free up a small amount of memory on your

machine to use for other things.

SDWRGMPD 1.10            Page 7 of 33

* /T

* /Table /Tbl /StatusTable /StatusTbl /Status

This pulls up a TABLE that shows the current status of SDWRGMPD as

it sits in memory.  It shows you the current settings of all of the

changeable values in SDWRGMPD.  The following shows you what you

would see if you did a "SDWRGMPD /T" with all values set at

default:

CURRENT INFORMATION FOR SDWRGMPD

��������������������������������

Status = Enabled

Coords =     3   50  100

��������������������������������

ALeft  = DpadLeft

ARight = DpadRight

AUp    = DpadUp

ADown  = DpadDown

A1     = Left

A2     = Right

��������������������������������

BLeft  = Y

BRight = C

BUp    = Z

BDown  = B

B1     = X, Start

B2     = A, M

��������������������������������

The first line of the TABLE indicates whether SDWRGMPD is DISABLEd

or ENABLEd (see pages 6 and 7, above, respectively).  The second

line shows you the coordinates that are returned by the BIOS when

you have the GamePad simulating an analog joystick handle movement

(see the CENTER Option below, page 9).  The first number (3) is

what will be returned when the joystick handle is Up or to the

Left, the middle number (50) is what will be returned when the

joystick handle is Centered, and the last number (100) is what will

be returned when the joystick handle is Down or to the Right.

The remaining lines of the TABLE show which digital (GamePad)

buttons are mapped to which analog values.  In this example, the

first line of the mappings, "ALeft = DpadLeft", indicates that the

{DPadLeft} button on the GamePad is mapped to the Left handle

movement of the analog A joystick.

The last line of the TABLE ("B2 = A, M"), indicates that both the

{A} and {M} buttons on the GamePad are mapped to the same analog

joystick button, B2.

The mapping Options below (page 12) describe this in more detail.

SDWRGMPD 1.10            Page 8 of 33

* /C:nn

* /Center:nn /Cntr:nn /Ctr:nn

This Option tells SDWRGMPD what coordinates to return when a

program calls the joystick BIOS routine and asks about the joystick

handles (whether they are centered or not).  The nn is a number you

must enter, and can be anything between 10 and 200.  The default is

50.

An analog game port has four analog coordinates that it can return

to the computer.  When you use real (analog) joysticks, you can

have two separate joysticks hooked up to your computer.  Two of the

analog coordinates are used for each joystick.  One of the analog

coordinates corresponds to the X (Horizontal or Left-Right)

location of the A joystick handle, one to the Y (Vertical or Up-

Down) location of the A joystick handle, and the other two analog

coordinates correspond to the X and Y location of joystick B

handle.

On an analog joystick, the coordinates are truly analog.  When the

joystick handle is centered, the standard BIOS usually returns a

coordinate of somewhere around 50.  It could be something more or

less than that, though, depending on what kind of computer you

have, who wrote the BIOS program of the computer, who made the game

port of the computer (and if it's a speed-adjustable game port,

what you have the speed adjusted to), what kind of joystick you

have, where you have the trimmers set on the joystick, and what the

temperature is inside your computer and inside the joystick.

Exactly what the number is normally doesn't make that much

difference.

When you move an analog joystick handle to the Left, the X

coordinate gets smaller.  It will constantly keep getting smaller

as you keep moving the handle to the Left.  When you finally have

the handle all the way to the Left, the BIOS will return a

coordinate of somewhere around 3 (exactly what the number is varies

from computer to computer, and it doesn't really matter what it is,

as long as it's not 0!).  As you move the handle to the Right, the

coordinates get bigger.  When the handle is all the way to the

Right, the coordinate will be about twice as big as when the handle

was centered (probably not exactly twice as big, but, again, it

doesn't really matter).

As you move the handle Up and Down, a similar thing happens.  When

the handle is all the way Up, the BIOS returns a Y coordinate that

is around 3.  When it is centered, it will be approximately 50.

When it is all the way Down, it will be somewhere around 100.

The program I have included with SDWRGMPD, called THRUST, will let

you see all of this stuff that your joystick is doing.  Check it

out.

SDWRGMPD 1.10            Page 9 of 33

A SideWinder GamePad doesn't have an analog handle, and doesn't

return analog values.  The DPad, which is supposed to be the

equivalent of an analog joystick handle, simply returns an "On" or

"Off" for each of the four possible directions.  SDWRGMPD converts

these On/Off buttons into something that looks like an analog

joystick coordinate, where it is either centered, Left/Right, or

Up/Down.  Of course, SDWRGMPD can only make it look like the handle

is either centered or all the way to one of the stops - it cannot

make it look like it is only partially moved the way an analog

joystick can.

The CENTER Option determines the coordinates that the BIOS function

will return when a program asks about the joystick.  You tell

SDWRGMPD the CENTER coordinate, and it automatically calculates the

Lower coordinate (Left or Up) and the Upper coordinate (Right or

Down).  SDWRGMPD always uses 3 as the Lower coordinate, the number

that you give it as the Center coordinate, and twice the number you

give it as the Upper coordinate.  The default coordinates are:

Lower:    3

Center:  50

Upper:  100

If you changed the CENTER to 10 (the minimum allowed), the

following coordinates would be used:

Lower:    3

Center:  10

Upper:   20

If you changed the CENTER to 200 (the maximum allowed), the

following coordinates would be used:

Lower:    3

Center: 200

Upper:  400

SDWRGMPD 1.10            Page 10 of 33

There are only two instances where you should need to use the

CENTER option.  The first is if you have a program that has "hard-

coded" what it thinks your joystick coordinates should be.  No

program SHOULD EVER do this, but I've seen a few that do.  Most

programs have a joystick calibration routine you go through, where

they will determine what the real Lower, Center, and Upper

coordinates for your particular joystick are.  Any program that has

a calibration routine should work fine with the default SDWRGMPD

coordinates.

The other instance where you might want to use the CENTER option is

in a program that has a joystick calibration routine, but will also

assume some "default" coordinates for your joystick that it thinks

may work even if you don't go through the calibration routine.

Most programs don't do this, either, but some do.  Most programs

are set up so that if you want to use a joystick at all, you have

to go through the entire joystick calibration routine each time you

start the program (which is usually a rather large pain in the

neck).

If you are using a program that will let you avoid the joystick

calibration routine and still use a joystick, and the default

coordinates returned by SDWRGMPD don't work correctly, you can set

up SDWRGMPD so that you can avoid having to go through the joystick

calibration routine.  Simply adjust the CENTER coordinate for

SDWRGMPD before you start the program to something that the program

likes, and then Reset the coordinate back again when the program is

completed.  See the batch file above (page 4) for an example of how

to do this.

SDWRGMPD 1.10            Page 11 of 33

The following Options are how you change the Button Mapping: which

digital buttons on the SideWinder GamePad are mapped to which analog

values in the joystick BIOS.

* /AU:� /AUp:�

* /AD:� /ADn:�   /ADown:�

* /AR:� /ARt:�   /ARight:�

* /AL:� /ALt:�   /ALft:�   /ALeft:�

* /AM:� /AMove:� /AMoves:� /AMovement:� /AMovements:�

* /A1:�

* /A2:�

* /AB:� /ABtn:� /ABtns:� /AButton:� /AButtons:�

* /BU:� /BUp:�

* /BD:� /BDn:�   /BDown:�

* /BR:� /BRt:�   /BRight:�

* /BL:� /BLt:�   /BLft:�   /BLeft:�

* /BM:� /BMove:� /BMoves:� /BMovement:� /BMovements:�

* /B1:�

* /B2:�

* /BB:� /BBtn:� /BBtns:� /BButton:� /BButtons:�

*  JoystickA  JoyA

*  JoystickB  JoyB

The first letter (A or B) or word (JoystickA, JoyA, JoystickB,

JoyB) of each Option indicates which analog joystick you are

referring to.  The second part of the word indicates either which

analog handle movement direction(s) (Up, Down, Left, Right) or

which analog joystick button(s) (1, 2) you are referring to.  The

information after the colon (the "�"), is the list of digital

buttons you want to map to that particular analog handle movement

or button.  I know this is a little confusing right now, so lets

look at each part in detail.

The four analog handle movement directions for each joystick are

called Up, Down, Left and Right (with various sorts of

abbreviations for them: Up = U or Up, Down = D or Dn or Down,

etc.).  You can also refer all four analog movement directions at

the same time with an M (or Move or Moves or Movement or

Movements).  For example, "ALeft" refers to the left handle

movement of analog joystick A, and "BMoves" refers to all four

handle movements of joystick B (BUp, BDown, BLeft, and BRight) at

the same time.

SDWRGMPD 1.10            Page 12 of 33

The two analog joystick buttons for each joystick are simply called

1 and 2.  You can also refer to both analog buttons at the same

time with a B (or Btn or Btns or Button or Buttons).  For example,

"B2" refers to the second button of analog joystick B, and

"AButtons" refers to both buttons on analog joystick A (A1 and A2)

at the same time.

The "�" part of the Option, which comes after the ":" or "=", is a

List of which digital GamePad buttons should be mapped to that

particular analog value.  The Button List is a list of one or more

of the digital buttons, which can be separated by spaces or commas.

Here is what you can call the digital buttons:

A BtnA ButtonA

B BtnB ButtonB

C BtnB ButtonB

M BtnM ButtonM

X BtnX ButtonX

Y BtnY ButtonY

Z BtnZ ButtonZ

Start BtnStart ButtonStart S BtnS ButtonS

R Rt Right BtnR BtnRt BtnRight          <Ŀ Buttons on front

ButtonR ButtonRt ButtonRight          <Ĵ of GamePad

L Lt Lft Left BtnL BtnLt BtnLft BtnLeft <Ĵ (pressed with

ButtonL ButtonLt ButtonLft ButtonLeft <�� index fingers)

DU DUp DpadU DpadUp

DD DDn DDown DpadD DpadDn DpadDown

DR DRt DRight DpadR DpadRt DpadRight

DL DLt DLft DLeft DpadL DpadLt DpadLft DPadLeft

NOTE: The {Mode} button on the SideWinder GamePad CANNOT be

programmed to do anything.  It simply changes the way the GamePad

works internally.

You can also tell SDWRGMPD to not map any digital buttons to a

particular analog value with one of the following as the digital

Button List (the "�"):

0 None Null Nada Nothing

The default mapping of the buttons is as follows.  The four

directions of the DPad correspond to the four directions of the

analog joystick A handle.  The (unlabeled) buttons on the {Left}

and {Right} side on front of the GamePad correspond to analog

buttons A1 and A2, respectively.  The {B}, {C}, {Y}, and {Z}

digital buttons, which are laid out in a T-pattern similar to the

DPad, correspond to the analog joystick B handle.  Digital buttons

{X} and {Start}, which are almost directly next to each other, both

correspond to analog button B1.  Digital buttons {A} and {M}

correspond to analog button B2.

SDWRGMPD 1.10            Page 13 of 33

The following on a single command line would give you the same

button mapping as the default:

SDWRGMPD /ALeft=DpadLeft      /ARight=DpadRight

/AUp=DpadUp          /ADown=DpadDown

/BLeft=Y  /BRight=C  /BUp=Z       /BDown=B

/A1=Left  /A2=Right  /B1=X,Start  /B2=A,M

I know you still must be at least a little overwhelmed and confused

by all of this, so let's go over some examples.  Let's say that

when you press the {A} button on the GamePad, you want a program

think that you pressed the analog A1 button.  You would do one of

the following:

SDWRGMPD /A1:A

SDWRGMPD /A1:BtnA

SDWRGMPD /A1=ButtonA

Note that when you map an analog value like this, the old map is

replaced, not added to.  For example, the default mapping for the

analog A1 button is the {Left} digital button (that is, a

"SDWRGMPD /A1:Left" would be the same as the default mapping).

When you do the "SDWRGMPD /A1:A", the mapping for the analog A1

button changes to digital button {A}.  It does not change to

digital button {A} AND digital button {Left}.  The {A} replaces the

{Left}, it does not add to it.

You can, however, map more than one digital button to any analog

value.  To do this, you give a complete list of the digital buttons

at one time, which can be separated by commas or spaces.  For

instance, to map three of the digital buttons, {A}, {B}, and {C},

to analog button B1, you would type one of the following:

SDWRGMPD /B1= A, B, C

SDWRGMPD /B1: ABC

SDWRGMPD /B1= A BtnB ButtonC

When you map more than one digital button to an analog value, only

one of the buttons has to be pressed for SDWRGMPD to return a

"analog button pressed" or "analog joystick handle moved" to the

program asking about the joystick.  You do not have to have all of

the buttons pressed at the same time.  In the above example, you

could have either the {A}, {B}, or {C} button pressed, or any two

of them, or even all three at the same time, and the program asking

about the joystick would think you have the analog B1 button

pressed.

SDWRGMPD 1.10            Page 14 of 33

SDWRGMPD allows you a lot of flexibility in how you can map the

digital buttons to the analog values.  In fact, SDWRGMPD allows you

so much flexibility that you can end up doing things that can get

you into trouble.  Let's go over some of them.

On a real analog joystick handle, and also on the DPad of the

SideWinder, it is physically impossible to do certain combinations

of things at the same time.  For example, it is impossible for a

joystick handle (or Dpad) to be both Up and Down at the same time

(they are made that way on purpose!).  It is also impossible for

them to be both Left and Right at the same time.

Most programs know this, and make no allowance for the possibility

that a joystick could be both Up and Down at the same time.  In

fact, if you did manage to do it, there's no telling how any

particular program might react.  Some programs may work OK, but

most will either ignore one of the directions or will crash the

computer.

With SDWRGMPD, you can map buttons in such a way that it is

possible to press both a Left and Right (or Up and Down) at the

same time.  For example, in the default mapping for SDWRGMPD, the

digital {Y} button is mapped to BLeft (the left movement of analog

joystick B), and digital button {C} is mapped to BRight.  It is

very possible for you to press both the {Y} (BLeft) and the {C}

(BRight) buttons at the same time, which is something that could

NEVER happen if you were using a real joystick.

Because doing something like this will very likely cause you some

problems, and because you could easily press a wrong button

combination by accident, SDWRGMPD handles the situation

automatically.  If you have both the Up and Down (or Left and

Right) buttons pressed at the same time, SDWRGMPD will act like

neither of the buttons is pressed.  It will tell the program asking

about the joystick that the joystick handle is centered.

SDWRGMPD 1.10            Page 15 of 33

It is also possible for you to map a single digital button to more

than one analog value.  For example, let's say you did the

following:

SDWRGMPD /A1=Start /A2=Start

When you press the {Start} button on the GamePad, SDWRGMPD will

tell the program asking about the joystick that you have both the

A1 and A2 buttons pressed at the same time.  This may, in fact, be

what you want to have happen.  You usually don't want to do

something like this, however, especially if you are mapping to

analog handle movements (instead of analog buttons like we did in

this example).  SDWRGMPD will not prevent you from mapping a single

digital button to more than analog value,  but it will beep at you

and put a warning message on the screen to let you know that what

you're doing could cause problems for you.

Let's say you did the following:

SDWRGMPD /ALeft=0

This tells SDWRGMPD to not map any button to the analog ALeft

handle movement.  However, you still have a button mapped to the

ARight movement (the default is {DPadRight}).  With no digital

button mapped to ALeft, SDWRGMPD will never be able to tell the

program asking about the joystick that the joystick A handle is

moved to the left.  The only things the program would ever see are

the Center and Right coordinates (50 and 100, respectively, if you

did not change the default CENTER).

Some programs may work OK by just seeing the 50 and the 100, but

many will not.  The joystick calibration routines of many programs

ask you to rotate the joystick handle around in a circle, then

center the joystick handle, and then press a button (on the

joystick or on the keyboard).  The calibration routine is expecting

to see at least three different coordinates (Left, Center, and

Right), and is also expecting one of the coordinates to be very

small (somewhere around 3).  If a program doesn't see what it is

expecting to see, it may think you have a bad joystick, or could

possibly even crash the computer.

There may be a very good reason you have for only mapping one half

of a joystick handle movement pair (Left/Right or Up/Down), and not

mapping the other half.  Therefore, SDWRGMPD will not prevent you

from mapping the buttons this way.  SDWRGMPD will, however, beep at

you and put a warning message on the screen to let you know that

what you're doing could cause problems for you.

SDWRGMPD 1.10            Page 16 of 33

Let's go over another possibility:

SDWRGMPD /BLeft=0 /BRight=0

This will completely turn off the analog joystick B Left/Right

movement.  When you turn off both halves of a handle-movement pair

like this (Left/Right or Up/Down), SDWRGMPD will tell the program

asking about the joysticks that there is no Left/Right B handle at

all.  That is, it will ALWAYS tell the program that the coordinate

is 0 (that it does not exist).  It will NOT tell the program that

the handle is always centered (normally, this would be 50).

Some programs become confused and do not know how to react if you

have two joysticks.  They expect you to only have one.  If you did

one the following:

SDWRGMPD /BLeft=0 /BRight=0 /BUp=0 /BDown=0 /B1=0 /B2=0

SDWRGMPD /BMovements=Null /BButtons=None

the program would think only one analog joystick (joystick A) was

hooked up to the computer, since you don't have anything mapped to

analog joystick B.

Let's say you did the following:

SDWRGMPD /A1=0

This eliminates the mapping for analog button A1 (by default, the

digital {Left} button was mapped to A1).  You actually did two

"unusual" things here, neither of which is considered an error or a

warning to SDWRGMPD.  There is now nothing mapped to analog button

A1, so there is no way to make the program think you pressed the A1

button.  Also, the digital {Left} button is not mapped to anything

any more - when you press it, nothing will happen.  Neither of

these is an error or a problem as far as SDWRGMPD is concerned.  It

could confuse you, though, and your program may not work the way

you think it should.

This may be somewhat obvious by now, but it's worth stating.  There

are 12 analog values that SDWRGMPD can "simulate", but there are 14

digital buttons on a SideWinder GamePad.  You don't have to map all

of the digital buttons, or all of the analog values, to anything at

all.  However, there are more digital buttons that there are analog

values, so you will always have at least two "spare" buttons to

work with.  You can either map them or not (by default, SDWRGMPD

maps every digital button to something).  It is up to you to make

sure that the buttons are mapped exactly the way you want them to

be.

SDWRGMPD 1.10            Page 17 of 33

* /O:{filename}

* /OptionsFileName:{filename} /OptionFileName:{filename}

* /OptionsFile:{filename} /OptionFile:{filename}

* /Options:{filename} /Option:{filename}

* /File:{filename} /FileName:{filename}

* /ConfigurationFileName:{filename} /ConfigurationFile:{filename}

* /Configuration:{filename}

* /ConfigureFileName:{filename} /ConfigureFile:{filename}

* /Configure:{filename}

* /ConfigFileName:{filename} /ConfigFile:{filename} /Config:{filename}

* /CfgFileName:{filename} /CfgFile:{filename} /Cfg:{filename}

* /InitializationFileName:{filename} /InitializationFile:{filename}

* /Initialization:{filename}

* /InitializeFileName:{filename} /InitializeFile:{filename}

* /Initialize:{filename}

* /IniFileName:{filename} /IniFile:{filename} /Ini:{filename}

This is how you can load Options to configure SDWRGMPD from a file,

instead of from the DOS command line (or from an Environment

variable).  The number of Options you can enter from a DOS command

line is limited to 126 characters, which may not be enough for you

to do everything you want.  Using an OPTIONSFILE will allow you to

make lots of changes (more than you can possibly do from the

command line), and also allows you the freedom to add comments to

help you organize and understand things better.

The {filename} part of the Option is the name of the OPTIONSFILE

you what SDWRGMPD to use.  It can also include a drive and path, if

necessary.  Here are some examples:

SDWRGMPD /O:SDWRGMPD.CFG

SDWRGMPD /ConfigFile = C:\UTIL\JOYSTICK\DEFAULT.SDW

The OPTIONSFILE itself must be a plain ASCII text file, and the

format resembles the Options as they are entered on the command

line.  Instead of separating the Options with the slash character

("/"), though, you put each Option on a separate line in the

OPTIONSFILE.  You can also have blank lines in the OPTIONSFILE, and

you can enter comments on a line by starting the comment with a

semicolon (";").  The following example compares a set of Options

on the command line to a similar set of Options in an OPTIONSFILE:

SDWRGMPD 1.10            Page 18 of 33

Command Line:

SDWRGMPD /Enable /ALeft=DPadLeft /ARight=DPadRight

OPTIONSFILE:

;This is a sample OPTIONS file for SDWRGMPD!

Enable              ;Enable and reset everything

ALeft  = DPadLeft   ;Joystick A, Move Handle Left

ARight = DPadRight  ;Joystick A, Move Handle Right

;End of SDWRGMPD OPTIONSFILE

You can include any of the standard Options in the OPTIONSFILE,

except to "nest" an OPTIONSFILE by telling one OPTIONSFILE to load

another OPTIONSFILE.  The OPTIONSFILE Option can only be issued

from the command line or from the SDWRGMPD Environment variable

(see "THE SDWRGMPD ENVIRONMENT VARIABLE" below, page 20).

The main purpose for an OPTIONSFILE is when you want to change a

whole bunch of things about SDWRGMPD at the same time, but the

limited length of the DOS command line prevents you from doing it

with a single SDWRGMPD command.  You really shouldn't need to

change SDWRGMPD Options very much, if at all, after it is

installed.  The most likely place you would use an OPTIONSFILE is

when you first install SDWRGMPD (in your AUTOEXEC.BAT file), and

even then only if you think the SDWRGMPD default settings are

totally ridiculous and you think every one of them needs to be

changed.

When you give SDWRGMPD the OPTIONSFILE {filename}, make sure you

include enough drive and directory (folder) information so that

SDWRGMPD knows for sure where to find the correct file.  I won't go

into a lot of detail here about drives and directories and files

file extensions, since you should already have a basic

understanding of how that stuff works.  I will say that it is

usually best to include the complete {filename}, which includes the

drive and the directory (like "C:\UTIL\SDWRGMPD.INI").  Not

including the drive and directory can prevent SDWRGMPD from being

able to find the appropriate OPTIONSFILE.

I have included a sample OPTIONSFILE with SDWRGMPD, called

SDWRGMPD.CFG.  Look it over, copy it, and edit it to create your

own OPTIONSFILEs.

SDWRGMPD 1.10            Page 19 of 33

THE SDWRGMPD ENVIRONMENT VARIABLE

���������������������������������

SDWRGMPD also looks for an environment variable called SDWRGMPD.  This

variable can have any of the standard command line Options listed above.

SDWRGMPD reads the Options from the environment variable SDWRGMPD before

it reads the Options from the DOS command line.  Whatever Options you

have for the SDWRGMPD environment variable effectively become your

"defaults".

This could be useful, for example, if you would normally like to not

have SDWRGMPD act like you have a second (B) analog joystick.  Instead,

you would normally like the {B}, {C}, {Y}, and {Z} buttons to simply

duplicate what the DPad does.  This will let you use either your right

or left hand to move the joystick handle, in case one of your hands gets

tired.  To do this, you could put the following line in your

AUTOEXEC.BAT file:

SET SDWRGMPD=/E /AL:DL,Y /AR:DR,C /AU:DU,Z /AD:DD,B /BM:0 /BB:0

If you did this, you would never have to worry about remembering how to

set everything back to YOUR "default" values again.  Every time you

simply typed "SDWRGMPD", it would act like you typed all of this other

stuff at the same time.

You could also combine the SDWRGMPD environment variable and an

OPTIONSFILE together, which could avoid the "cryptic" line you see above

(which is more than a little bit hard to understand).  You could create

an OPTIONSFILE similar to the following (let's call it

"C:\UTIL\JOYSTICK\SDWRGMPD.CFG"):

;Sample C:\UTIL\JOYSTICK\SDWRGMPD.CFG file

Enable

JoystickALeft      = DPadLeft,  ButtonY

JoystickARight     = DPadRight, ButtonC

JoystickAUp        = DPadUp,    ButtonZ

JoystickADown      = DPadDown,  ButtonB

JoystickBMovements = Null

JoystickBButtons   = Null

and then put a line similar to the following line in AUTOEXEC.BAT:

SET SDWRGMPD=/OptionFile:C:\UTIL\JOYSTICK\SDWRGMPD.CFG

If you don't know what an "environment" is, or what it is used for, I

suggest you read about it in your favorite DOS book.

SDWRGMPD 1.10            Page 20 of 33

Also, just so you know, DOS sometimes does not let you SET environment

variables that contain an equals "=" anywhere in them.  However, in the

command line Options for SDWRGMPD, or in an OPTIONSFILE, you can use ":"

and "=" interchangeably.  On the command line, you can use either ":" or

"=", but in the SDWRGMPD environment variable you may need to use ":".

Again, this is a limitation of certain versions of DOS, not of SDWRGMPD.

SDWRGMPD 1.10            Page 21 of 33

ERRORLEVELS

�����������

SDWRGMPD sets the following ERRORLEVELs after it executes, to give you

an indication of what happened.  If you don't understand the purpose or

value of ERRORLEVELs, I suggest you read about them in your DOS manual.

������������������������������������������������������������������ͻ

� ERRORLEVEL �                   MEANING                           �

������������������������������������������������������������������͹

�     0      � No problem - SDWRGMPD was installed or updated OK   �

�     1      � Couldn't understand one or more of the Options      �

�     2      � Couldn't find a good SideWinder GamePad             �

�     3      � Memory problem with the computer                    �

�     4      � Tried UNINSTALL without SDWRGMPD installed          �

�     5      � A different version of SDWRGMPD was found in memory �

������������������������������������������������������������������ͼ

SDWRGMPD 1.10            Page 22 of 33

SDWRGMPD & WINDOWS

������������������

The Win95 software that Microsoft provides with the SideWinder is

incompatible with SDWRGMPD.  That is, they both cannot be enabled and

running at the same time.  The Microsoft software will work, but it

"hogs" the GamePad and will prevent SDWRGMPD from working properly.

If you have the Microsoft software installed, and want to run a DOS

program in a DOS box, and also want to use SDWRGMPD, you will have to

disable the Microsoft software.  Unfortunately, Microsoft does not

provide a way to "disable" their software.  If the software is

installed, and the GamePad is hooked up to the computer, the software

will automatically keep polling the GamePad (and keep interfering with

SDWRGMPD).  There is no way to turn it off, short of removing the

Microsoft software.

There are two ways to temporarily remove the Microsoft software (so that

you can install it again fairly easily).  The first is to get into:

Start -> Settings -> Control Panel -> Game Controllers

and then Remove the GamePad.  Run the program (in a DOS box).  When you

are done with the program, get back into:

Start -> Settings -> Control Panel -> Game Controllers

and Add the GamePad back in again.

The other way to temporarily remove the Microsoft software is to get

into:

Start -> Shut Down -> Restart the computer in MS-DOS mode.

This does sort of a half-reboot of the computer, where none of the

Windows software is installed (in particular, the Microsoft GamePad

software is not installed).  Run the DOS program (it will not be in a

DOS box any more, but will be running almost like you booted straight to

DOS).  When you're done, type "EXIT" at a DOS prompt and Win95 will

start up again.

SDWRGMPD 1.10            Page 23 of 33

I understand that this is much more complex and confusing than it should

be to run a simple DOS program (you can blame Microsoft for that one,

though, not SDWRGMPD).  In case you didn't know already, Microsoft

doesn't want you to use DOS any more.  Every new version of Microsoft

software that comes out is bigger, slower, and less compatible with

older programs than the previous versions were.  They are even

manufacturing new hardware and never intending to make it compatible

with DOS programs, no matter how good the DOS programs are.  Microsoft

is also encouraging (or forcing) other manufacturers to do the same.

In fact, Microsoft would like you to believe that it is IMPOSSIBLE to

use their new hardware with anything other than the latest version of

Windows, which is an outright lie.  Just because Microsoft doesn't make

a driver for a particular piece of hardware does not mean one can't (or

won't) be written.  It just means that Microsoft won't do it themselves,

and it sucks.

SDWRGMPD 1.10            Page 24 of 33

OTHER DIGITAL JOYSTICKS

�����������������������

Digital joysticks are relatively new to the computer world.  It is

almost guaranteed that any program more than a couple of years old will

not know about digital joysticks.  If the program is continuing to be

updated, the manufacturer may have added digital joystick support (at

least for some digital joysticks).  The problem is, every digital

joystick is different.  Even just within the Microsoft SideWinder line

of hardware, there are about a half-dozen different versions.  They all

have different numbers of buttons, Microsoft has different names for the

buttons, and they all "talk" to the computer in a little bit different

way than the other ones do.

SDWRGMPD works with a single Microsoft SideWinder GamePad.  It has not

been tested with, and probably will not work with, any device other than

a single Microsoft SideWinder GamePad.  Microsoft claims you can cascade

up to four SideWinder GamePads together.  If you have more than one

GamePad hooked up at the same time, though, SDWRGMPD will not work (it

will think the GamePad is broken).

The SideWinder GamePad also has a {Mode} button, that effectively

toggles the GamePad on and off.  When it is off, you can have an analog

joystick cascaded onto the GamePad, and you can use the analog joystick

like normal.  Rather than plugging and unplugging joysticks, you can

simply disable the GamePad with the {Mode} button.  Of course, if

Microsoft would write a DOS driver for their joysticks, there would

probably be no need to even have a {Mode} button!

I suspect most of the other Microsoft SideWinder Digital products use

the same basic protocol to talk to the computer as the SideWinder

GamePad.  I don't know this for sure, though, and really don't have the

money (or time) to buy all of the different products, test them, and

build special drivers for them.  Microsoft doesn't publish the

specifications for the SideWinder protocol, so it's necessary to buy one

of the products and try to decipher the protocol yourself.  If someone

out there has another SideWinder product, and wants a DOS driver for it,

we might be able to arrange something.  At a minimum, letting me borrow

the joystick for awhile (if you live in the Denver area, at least), or

helping me to buy one, could definitely make things happen more quickly.

SDWRGMPD 1.10            Page 25 of 33

There are also other manufacturers that make digital joysticks

(Logitech, CH Products, and ThrustMaster, to name a few).  It seems that

every manufacturer invents their own proprietary protocol, and the only

drivers they make are for the latest version of Windows.  I've never

seen another DOS driver for any digital joystick.  I'm not saying there

aren't any, but I've never seen one, and I don't ever expect to ever see

one come from a joystick manufacturer.  That's pretty pathetic, if you

ask me.

I've recently discovered a web site that contains a veritable gold mine

of information about digital joysticks from different manufacturers.

The site contains the source code for the joystick drivers that are

included with Linux.  The list joysticks supported by Linux is quite

impressive, especially considering that the joystick manufacturers are

either reluctant or totally non-responsive in providing documentation

about the digital protocols.  The URL of the site is:

http://atrey.karlin.mff.cuni.cz/~vojtech/joystick

SDWRGMPD 1.10            Page 26 of 33

TROUBLESHOOTING

���������������

There are five basic reasons why SDWRGMPD may not install itself into

memory, or may not work properly after it is installed:

1. You do not have a single SideWinder GamePad plugged into your game

port.  It is unplugged, it is not a SideWinder GamePad, or you have

more than one of them cascaded together.

2. The GamePad is plugged in properly, but the {Mode} button has been

pressed (effectively disabling the GamePad).  Press the {Mode}

button until the Mode light is on.

3. You have a bad game port.  Even though game ports are relatively

simple to build, manufacturers still manage to screw them up.  A

SideWinder requires a fully functional game port, capable of

handling two analog joysticks.  Many game ports are only capable of

handling one analog joystick.

4. Your computer is too slow.  Microsoft could have, and should have,

made the GamePad capable of working on any computer, but they

didn't.  I do not know what the minimum required computer speed is.

I do know that a 4.77MHz 8088 is too slow, and that a 12MHz 286

works just fine.  Slow 286's or even fast 8088's may also work, but

I don't know for sure (I'd appreciate feedback from anyone who

tries it)!

5. You are running in a Win95 DOS box, but have not disabled (removed)

the Win95 GamePad software.  SDWRGMPD will not work if the

Microsoft GamePad software is running at the same time.  See the

"SDWRGMPD & WINDOWS" section above (page 23) on how to resolve

this.

There are four reasons why a particular program will not work with

SDWRGMPD:

1. The program looks at the joystick itself directly, and does not use

the BIOS to determine the joystick coordinates.  Such a program

will have to add internal digital joystick support to work with ANY

digital joystick (not just a SideWinder GamePad), especially if the

program will not run in a Win95 DOS box.

2. SDWRGMPD was never installed into memory, or was UNINSTALLed.  You

can install it by simply typing "SDWRGMPD" at a DOS prompt.

3. SDWRGMPD is installed, but has been DISABLEd.  Re-ENABLE it by

typing "SDWRGMPD /E" at a DOS prompt.

4. The digital GamePad buttons are not mapped correctly to the analog

values the program is expecting to see.  Type "SDWRGMPD /T" to see

how the buttons are mapped.

SDWRGMPD 1.10            Page 27 of 33

I have also included with SDWRGMPD a program called THRUST.  It is a

simple, low-level troubleshooting tool for you game ports and joysticks.

You can use it to help you configure and troubleshoot SDWRGMPD so that

it will work like you want it to.  It includes its own documentation, so

I won't discuss how to use it here.  Just realize that it's available,

and it's VERY handy for anyone who has a joystick (analog or digital).

SDWRGMPD 1.10            Page 28 of 33

A WORD FROM THE SPONSOR

�����������������������

SDWRGMPD is a free program.  If you like it and use it, do something

nice for someone else in return.  I will accept niceties to myself in

the form of $.  The joystick manufacturers should create programs like

SDWRGMPD and distribute them freely with their joysticks, just like they

distribute their Win95 drivers.  It's bad enough that they don't make

non-Windows drivers, but then they go a step further and don't even

provide the documentation needed for others to create drivers.  I find

it totally despicable.

You can freely copy and distribute SDWRGMPD.COM, as long as it is

distributed along with this SDWRGMPD.DOC, and neither file has been

modified in any way.  You cannot charge anyone in any way for SDWRGMPD,

except to recover your actual costs for disks and shipping (you cannot

charge for "handling").  You do need my permission to distribute

SDWRGMPD as a "companion" to some other program.

You can also freely download the source code for SDWRGMPD if you want.

It is available from my Home Page, http://members.aol.com/bretjohn.  I

don't include it with the program to keep the size of the download as

small as possible, and because most people don't care about the source

code anyway.

If you do download the source code, and use it to create your own driver

for another digital joystick, you must also do the following:

*  Make your driver freely available, just like SDWRGMPD is.

*  Attempt to upload it to some of the major freeware/shareware sites

on the Internet.

*  Send me a copy, so I can put it on my Home Page, or at least give

me a link to your Home Page.

*  Mention my name in your documentation.

*  Include the special BIOS function I've added to SDWRGMPD, that will

allow other programs to automatically determine if a special

(digital) joystick driver has been loaded.

The latest version of my JOYKEYS program, in particular, knows about

this special BIOS function.  JOYKEYS normally looks at the joystick

directly, since the standard joystick BIOS functions of most computers

are too slow and unreliable for a TSR like JOYKEYS to use.  JOYKEYS

calls the special BIOS function while it is installing.  If the function

works (indicating that a "special" joystick driver has been installed),

JOYKEYS uses the BIOS to look at the joystick instead of looking at the

joystick directly.  That way, JOYKEYS does not have to contain knowledge

about every conceivable type of digital joystick, which would make

JOYKEYS horrendously large (the way all Windows programs are nowadays).

SDWRGMPD 1.10            Page 29 of 33

The format of the special BIOS function is:

INT 15h, with AH=84h and DX=-1 (FFFFh)

If the driver is installed and ENABLEd, return with Carry clear and

DX=0.

If the driver is not installed, or is DISABLEd (which is functionally

the same as not having it installed), return with Carry set and AH=80h

or 86h (the regular BIOS function should do this for you automatically).

The regular BIOS will also usually return with DX unchanged, but you

should not depend on it.  The regular BIOS function may also change BX

and CX.

I would like to thank Robert William Grubbs.  He "decoded" the

SideWinder GamePad protocol and published it (something that Microsoft

still hasn't done).  I did not use his source code to create SDWRGMPD,

but I did use his documentation of the protocol.  Thank you very much!

SDWRGMPD 1.10            Page 30 of 33

OTHER PROGRAMS

��������������

If you like SDWRGMPD, I have written some other programs you may find

useful.

CLOCK

Shows a clock on your screen all the time.  Also has two alarms

that beep at you when it's time for an appointment.

JOYKEYS

Lets you use your joystick with ANY program.

MOUSKEYS

Lets you use your mouse with ANY program.

PRTSCR

Sends all of your PrintScreens to a file instead of a printer.  A

good way to transfer data between two programs, even if they won't

normally "talk" to each other.

PRTSCRFF

Modifies your PrintScreen so that each new PrintScreen gets put on

a new sheet of paper (especially useful if you have a Laser

printer).

SCANCODE

Type keystrokes automatically in the middle of other programs.

Helpful in getting past the "starting screens" of most programs, or

to create macros.  Can also be used to remap the keyboard.

SERIAL

Shows the status of your serial (COM) ports on screen.  Shows the

bit rate, parity, status of the leads (DTR, DSR, etc.).  An

excellent way to troubleshoot modem problems.

SLOWDOWN

Slows down a fast computer so you can run older programs that

require a PC/XT or PC/AT.

SDWRGMPD 1.10            Page 31 of 33

WARRANTY

��������

There is no warranty of any kind, either expressed or implied, supplied

with SDWRGMPD.  As with any software, especially TSR's, it may or may

not work with other software that you may have.  I have done my best to

make sure SDWRGMPD won't screw anything up, but, again, there is no

guarantee.

If you find any "bugs" in this program, or see something that you wish

were different, please let me know.  I can only improve SDWRGMPD if I

know what you think needs to improve.

HAVE FUN!

Bret Johnson

6775 South Logan Street

Littleton, CO  80122-1249

(303) 795-5084

bretjohn@juno.com  or

bretjohn@aol.com

I intend to always keep the latest versions of all of my programs on my

own web page.  The URL is:

http://members.aol.com/bretjohn

Keep looking there for updates to my programs.

SDWRGMPD 1.10            Page 32 of 33

REVISION HISTORY

����������������

1999/05/20  v1.00

1999/11/08  v1.10

* Added the OPTIONSFILE Option, allowing a configuration file to be

used to change settings instead of just command line and Environment

variable switches.

* Allowed UNINSTALL and HELP to be in the SDWRGMPD Environment

variable.

* Added the synonyms JOYSTICKA & JOYA when referring to Joystick A,

etc.

* Changed a couple of items that should let SDWRGMPD work on extremely

fast computers (in the 1000MHz range, if they ever get that far and

still remain capable of running DOS).

* Various small reliability-enhancing changes.

SDWRGMPD 1.10            Page 33 of 33